Exercise: Identification of a First-Order System

A unit step input is applied to a system. The measured output is given below:

| t(s) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(t) | 0 | 0.39 | 0.63 | 0.78 | 0.86 | 0.92 | 0.95 | 0.97 | 0.98 |

Questions :

- Determine the static gain 𝐾

- Determine the time constant 𝜏.

- Give the transfer function of the system.

Exercise N°2: Let's redo this exercise using the Broïda method to identify the system with delay. In the course, we assume that the system can be modeled by: Fs=Ke-Tsτs+1

- Determine the static gain 𝐾

- Determine the time constants T and 𝜏.

- Give the transfer function of the system.

Exercise N°3: Homework: Solve the same exercise using the Strejc model.

Exercise N°4: Identification of a Second-Order System

A unit step input is applied to a system. The measured output is given below:

| t(s) | 0 | 0.5 | 1 | 1.5 | 2 | 2.5 | 3 | 3.5 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(t) | 0 | 0.3 | 0.6 | 0.9 | 1.5 | 1 | 0.95 | 0.95 | 0.95 |

- Determination of the static gain 𝐾.

- Determine the estimated damping coefficient  ζ.

- Determine the estimation of the natural frequency 𝜔0

- Identify the transfer function of this system.