Exercise N°1:

Consider the diagrams in the following figures:

- Determine the transfer function of each controller:  Hi(p) = 𝑆𝑖(𝑝)/𝐸𝑖(𝑝)

- Determine the type (nature) of each controller.

Exercise N°2:

Consider the process: G(p)=2/(2p+1)​

and the PI controller:  Gr (p)=𝐾𝑟(1+1/𝜏p) , with  𝐾r = 4 and  𝜏 = 2 s.

Study individually the effect of the parameters 𝐾𝑟 and τ (while keeping the other parameter constant) on the poles of the closed-loop system.

Exercise N°3:

Consider a second-order system with the transfer function: Fp= 1p2+1.6p+2

By using a proportional controller defined as (𝑝)=𝐾𝑝

- Give, in terms of 𝐾p, the expressions of: KCL, ωnCL and ξCL	​

- Determine the steady-state error for a step input of amplitude 10.

- What value of 𝐾p is necessary to achieve a steady-state error  of 0.01.

- Calculate the steady-state error if the controller  C(s) is replaced by a PI controller with transfer function:    Cp=Kp(1+1τp)