SERIES 2: Mechanics of Multibody Systems

Exercise N°1:

A system consists of three particles with masses m1= 3g located at (1,0,-1), m2= 5g located at (-2,1,3), and m3= 2g located at (3,-1,1). Find the coordinates of their center of mass.

Exercise N°2:

Two particles with masses m1 and m2​ are in motion. Show that the total kinetic energy of the system composed of the two masses is given by: T=12μv2+12Mv2   where:

- M=m1+m2 ​ is the total mass,

- μ=m1.m2m1+m2is the reduced mass of the system,

- Vis the velocity of the center of mass, and

- v=v1−v2 is the relative velocity between the two masses .

Exercise N° 3:

Find the centroid of a semi-circular surface with radius a.

Exercise N°4:

Suppose that the internal forces of a system of particles are conservative and are derived from a potential

Vλν(rλν)=Vνλ(rνλ)

Where rλν=rνλ=(xλ-xν)2+(yλ-yν)2+(zλ-zν)2 is the distance between particles λ and ν

- Prove that νλfλν.drν=-12νλdVν where fλνis the internal force on particle ν due to particle λ

- Evaluate the sum νλ12fλν.drν

Exercise N°5:

Find the centroid of an homogeneous hemisphere with radius a. Consider a simple integral by summing the disks of radius r from z=0 to z=a (see the figure below).

Find the centroid of a homogeneous solid composed of a hemisphere with radius a and a cylinder with base radius a and height h. It is given that M1=23πσa3 and M2= πσa2H.

Exercise N°6:

A uniform plate is bounded by the parabola y=x2 and the line y=H in the xOyx plane. Determine the position of the center of mass."