Université Djialli Bounaama de Khemis Miliana/Faculté des Sciences et de la Technologie

Département de Technologie     Niveau et Spécialité: Master 2/Systèmes de Télécom

Année universitaire :2017/2018    Semestre 3       Matière: Télévision  numérique    A.BOUNIF

TP N0 2: Etude, Implémentation et évaluation du MPEG2 sous Matlab

- Introduction : La norme MPEG2 est une norme internationale pour la mise en forme des signaux vidéo (image) et audio(Son) des services de télévision numérique. Elle est utilisée pour les services gratuits en définition standard.

MPEG = Motion Picture Expert Group (Groupe d’Experts de l’Image Animée), groupe international d’ingénieurs de recherche et développement qui a mis au point une série de normes et recommandations techniques pour la compression des signaux audio et vidéo numériques sans perte de qualité dans la perception des signaux restitués.

- Les principes de bases de la compression :

Dans la norme CCIR 601, puis dans le 4.1.1 et le 4.2.0, chaque pixel est codé indépendamment des autres. L’idée à la base de tous les systèmes de compression vidéo (mais aussi audio) est d’exploiter les redondances qui existent naturellement dans des images vidéo, voire de les éliminer pour réduire la quantité et le débit d’informations à transmettre.

Ces redondances sont de quatre ordres :

- Les redondances spatiales : dans les grandes plages uniformes à l’intérieur d'une image, il existe de nombreuses similitudes entre des points voisins.

- Les redondances temporelles : dans une séquence vidéo, les différences entre deux images successives sont minimes. On ne va coder que les différences entre des images successives et ne transmettre que les changements.

- Les redondances subjectives : il est inutile de coder les détails fins que l’œil ne voit pas.

- Les redondances statistiques : on utilise des traitements purement informatiques qui exploitent par exemple les similitudes dans les suites de valeurs numériques.

Ne pas transmettre un élément déjà transmis.

Ne pas transmettre ce que l’on ne voit pas ou ce que l’on n’entend pas.

- La séquence MPEG. Les différents types d’images :

Une séquence vidéo à la norme MPEG-2 peut être composée de trois types d’images :

- les images Intra (I)      - les images Prédites (P)       - les images bidirectionnelles (B)

Toutes ces images ne sont pas traitées et compressées de la même façon.

%%% …………………………………………………

mri = uint8(zeros(128,128,1,27));

for frame=1:27

[mri(:,:,:,frame),map] = imread('mri.tif',frame);

end

mov = immovie(mri,map);

implay(mov);

Manipulation des séquences  vidéo

- Exécuter le script Matlab ci-contre :

- Quelle est la tâche réalisée ?

- Afficher l’image traitée.

- Commenter chaque ligne de ce pg

- Taper le 2eme script Matlab.

- Visualiser le résultat de simulation.

- Commenter chaque ligne de ce pg.

- Quelle est la tâche réalisée ?

%%%%%% …………………………………………………………………………

trafficObj = mmreader('traffic.avi'); % …………………………………………………………………………

get(trafficObj) % …………………………………………………………………………

implay('traffic.avi'); % …………………………………………………………………………

darkCarValue = 50; % …………………………………………………………………………

darkCar = rgb2gray(read(trafficObj,71)); % …………………………………………………………………………

noDarkCar = imextendedmax(darkCar, darkCarValue); % …………………………………………………………………………

imshow(darkCar) % …………………………………………………………………………

figure, imshow(noDarkCar) % …………………………………………………………………………

sedisk = strel('disk',2); % …………………………………………………………………………

noSmallStructures = imopen(noDarkCar, sedisk); % …………………………………………………………………………

imshow(noSmallStructures) % …………………………………………………………………………

nframes = get(trafficObj, 'NumberOfFrames'); % …………………………………………………………………………

I = read(trafficObj, 1);

taggedCars = zeros([size(I,1) size(I,2) 3 nframes], class(I)); % ………………………………………………

for k = 1 : nframes

singleFrame = read(trafficObj, k); % …………………………………………………………………………

% Convert to grayscale to do morphological processing.

I = rgb2gray(singleFrame); % …………………………………………………………………………

% Remove dark cars.

noDarkCars = imextendedmax(I, darkCarValue); % …………………………………………………………………………

% Remove lane markings and other non-disk shaped structures.

noSmallStructures = imopen(noDarkCars, sedisk); % …………………………………………………………………………

% Remove small structures.

noSmallStructures = bwareaopen(noSmallStructures, 150); % …………………………………………………………………

% Get the area and centroid of each remaining object in the %frame. The object with the largest area is the light-colored % car.  Create a copy of the original frame and tag the car %by changing the centroid pixel value to red.

taggedCars(:,:,:,k) = singleFrame; % …………………………………………………………………………

stats =regionprops(noSmallStructures,{'Centroid','Area'});

if ~isempty([stats.Area])

areaArray = [stats.Area];

[junk,idx] = max(areaArray);

c = stats(idx).Centroid;

c = floor(fliplr(c));

width = 2;

row = c(1)-width:c(1)+width;

col = c(2)-width:c(2)+width;

taggedCars(row,col,1,k) = 255;

taggedCars(row,col,2,k) = 0;

taggedCars(row,col,3,k) = 0;

end

end

%Run the following commands to see taggedCars in implay.

frameRate = get(trafficObj,'FrameRate');

implay(taggedCars,frameRate);

- Exemple d’un codage MPEG-2

4 Estimation du mouvement en Matlab

1. Pour commencer, il faut extraire les contenu du fichier motion estimation.zip dans le r´epertoire de travail.

2. Lancez Matlab et ouvrez script me demo.m

3. Ex´ecutez ce script, et v´erifiez le r´esultat

4. Modifiez les param`etres de l’estimation du mouvement (taille des blocs, fenˆetre de recherche), et v´erifiez le

r´esultat sur : qualit´e de la pr´ediction, temps de calcul, coˆut de codage des vecteurs. Quelle est la configuration

optimale ?

5. Quelle est la strat´egie de recherche impl´ement´e dans cette exemple ?

% Example of MPEG-2 style coding. This function will get a movie, encode

% it, decode it, and write the MPEG data and both movie versions to disk

% for later playback and analysis. (See companion functions 'playlast' and

% 'quiverplot'.)% % By Steve Hoelzer % 2005-4-18

fprintf('\nMPEG Project\n')

nf = 10; % Number of frames to process, 0 = process entire movie

fprintf('%i frames\n',nf)

mov = getmov(nf);

tic

mpeg = encmov(mov);

fprintf('Encode time: %s\n',sec2timestr(toc))

tic

mov2 = decmpeg(mpeg);

fprintf('Decode time: %s\n',sec2timestr(toc))

save lastmov mov mpeg mov2

implay(mov2)

%

% ECE 434, Multimedia Communication Networks

% Professor Marilyn Andrews

% University of Illinois at Chicago (UIC)

- Essayer de simuler le programme ci-contre.

- Combien de fonction a-t-on besoin pour faire

fonctionner un projet MPEG-2?

- On vous donne une liste des fonctions, pour choisir

celles qui sont nécessaires, comme suit :

function rgb = convertYuvToRgb(yuv)

% convert row vector YUV [0, 255]

%in row vector RGB [0, 255]

% load conversion matrices

load conversion.mat;

yuv = double(yuv);

yuv(:, 2 : 3) = yuv(:, 2 : 3) - 127;

rgb = (yuvToRgb *yuv.').';

rgb = uint8(clipValue(rgb, 0, 255));

%%==============================================

function varargout = figuresc(sf)

% figuresc(sf) creates a normal figure except that figure size is controlled by

% the scaling factor, sf. The scaling factor is relative to screen size so 1.0

% makes the figure fill the entire screen. Note that the display portion of the

% window will be the same size as the screen. The window border will be outside

% of that so sf = 1.0 usually doesn't look good. Smaller scale factors adjust

% height and width proportionally and center the figure.

%

% The scaling factor can be a single value (sf = 0.8) to scale width and height

% identically or two values (sf = [0.8, 0.6]) to scale width and height

% seperately. The scaling factor(s) must be between 0.0 and 1.0.

%

% Demo code (and fun too!):

%      for sf = 0.9:-0.1:0.1, figuresc(sf), end

%

% Steve Hoelzer

% 2004-Sep-02 - Created

% 2004-Sep-03 - Option to scale width and height independently

% 2004-Oct-22 - Pass out figure handle if requested

% 2005-Mar-16 - Don't show figure handle on command line

% Error checking

if any(sf < 0.0) | any(sf > 1)

error('Scaling factor must be between 0.0 and 1.0')

end

% Scale width and height identically if sf has a single element

if numel(sf) == 1

sf = [sf sf];

end

% Calculate [left, bottom, width, height]

pos = [(1-sf)/2, sf];

% Display figure

f = figure('Units','Normalized', ...

'Position',pos(:)); % pos is always a vector

if nargout > 0

varargout{1} = f;

end

% function mov = loadFileYuv(fileName, width, height, nrFrame)

%%%%%%%%%%%%=============================================================

function [mov,imgRgb] = loadFileY4m(fileName, width, height, nrFrame)

% load RGB movie [0, 255] from YUV 4:2:0 file

fileId = fopen(fileName, 'r');

subSampleMat = [1, 1; 1, 1];

dummy = fgetl(fileId); % Skip file header

progressbar

for f = 1:nrFrame

f

% Skip frame header

dummy = fgetl(fileId);

% read Y component

buf = fread(fileId, width * height, 'uchar');

imgYuv(:, :, 1) = reshape(buf, width, height).'; % reshape

% read U component

buf = fread(fileId, width / 2 * height / 2, 'uchar');

buf = reshape(buf, width / 2, height / 2).';

imgYuv(:, :, 2) = kron(buf, subSampleMat); % reshape and upsample

% read V component

buf = fread(fileId, width / 2 * height / 2, 'uchar');

buf = reshape(buf, width / 2, height / 2).';

imgYuv(:, :, 3) = kron(buf, subSampleMat); % reshape and upsample

% convert YUV to RGB

imgRgb = reshape(convertYuvToRgb(reshape(imgYuv, height * width, 3)), height, width, 3);

mov(f) = im2frame(imgRgb);

progressbar(f/nrFrame)

end

fclose(fileId);

% function mov = loadFileYuv(fileName, width, height, idxFrame)

%%%%%%%%%%%%=================================================================

function [mov,imgRgb] = loadFileYuv(fileName, width, height, idxFrame)

% load RGB movie [0, 255] from YUV 4:2:0 file

fileId = fopen(fileName, 'r');

subSampleMat = [1, 1; 1, 1];

nrFrame = length(idxFrame);

sizeFrame = 1.5 * width * height;

progressbar

for f = 1 : 1 : nrFrame

f

% search fileId position

fseek(fileId, (idxFrame(f) - 1) * sizeFrame, 'bof');

% read Y component

buf = fread(fileId, width * height, 'uchar');

imgYuv(:, :, 1) = reshape(buf, width, height).'; % reshape

% read U component

buf = fread(fileId, width / 2 * height / 2, 'uchar');

imgYuv(:, :, 2) = kron(reshape(buf, width / 2, height / 2).', subSampleMat); % reshape and upsample

% read V component

buf = fread(fileId, width / 2 * height / 2, 'uchar');

imgYuv(:, :, 3) = kron(reshape(buf, width / 2, height / 2).', subSampleMat); % reshape and upsample

% normalize YUV values

% imgYuv = imgYuv / 255;

% convert YUV to RGB

imgRgb = reshape(convertYuvToRgb(reshape(imgYuv, height * width, 3)), height, width, 3);

% imgRgb = ycbcr2rgb(imgYuv);

%imwrite(imgRgb,'ActualBackground.bmp','bmp');

mov(f) = im2frame(imgRgb);

%   mov(f).cdata = uint8(imgRgb);

%   mov(f).colormap =  [];

%     imwrite(imgRgb,'ActualBackground.bmp','bmp');

%figure, imshow(imgRgb);

%name = 'ActualBackground.bmp';

%Image = imread(name, 'bmp');

%figure, imshow(Image);

progressbar(f/nrFrame)

end

fclose(fileId);

%=========================================================

function playlast(n)

if isempty(n)

n = 5;

end

load lastmov

for i = 1:size(mov,4)

m(i).cdata = uint8([mov(:,:,:,i) mov2(:,:,:,i)]);

m(i).colormap = [];

end

figuresc([0.9 0.5])

movie(m,n,10)

%%%%%%%%%==============

function progressbar(fractiondone, position)

% Description:

%   progressbar(fractiondone,position) provides an indication of the progress of

% some task using graphics and text. Calling progressbar repeatedly will update

% the figure and automatically estimate the amount of time remaining.

%   This implementation of progressbar is intended to be extremely simple to use

% while providing a high quality user experience.

%

% Features:

%   - Can add progressbar to existing m-files with a single line of code.

%   - The figure closes automatically when the task is complete.

%   - Only one progressbar can exist so old figures don't clutter the desktop.

%   - Remaining time estimate is accurate even if the figure gets closed.

%   - Minimal execution time. Won't slow down code.

%   - Random color and position options. When a programmer gets bored....

%

% Usage:

%   fractiondone specifies what fraction (0.0 - 1.0) of the task is complete.

% Typically, the figure will be updated according to that value. However, if

% fractiondone == 0.0, a new figure is created (an existing figure would be

% closed first). If fractiondone == 1.0, the progressbar figure will close.

%   position determines where the progressbar figure appears on screen. This

% argument only has an effect when a progress bar is first created or is reset

% by calling with fractiondone = 0. The progress bar's position can be specifed

% as follows:

%       [x, y]  - Position of lower left corner in normalized units (0.0 - 1.0)

%           0   - Centered (Default)

%           1   - Upper right

%           2   - Upper left

%           3   - Lower left

%           4   - Lower right

%           5   - Random [x, y] position

%   The color of the progressbar is choosen randomly when it is created or

% reset. Clicking inside the figure will cause a random color change.

%   For best results, call progressbar(0) (or just progressbar) before starting

% a task. This sets the proper starting time to calculate time remaining.

%

% Example Function Calls:

%   progressbar(fractiondone,position)

%   progressbar               % Initialize/reset

%   progressbar(0)            % Initialize/reset

%   progressbar(0,4)          % Initialize/reset and specify position

%   progressbar(0,[0.2 0.7])  % Initialize/reset and specify position

%   progressbar(0.5)          % Update

%   progressbar(1)            % Close

%

% Demo:

%   n = 1000;

%   progressbar % Create figure and set starting time

%   for i = 1:n

%       pause(0.01) % Do something important

%       progressbar(i/n) % Update figure

%   end

%

% Author: Steve Hoelzer

%

% Revisions:

% 2002-Feb-27   Created function

% 2002-Mar-19   Updated title text order

% 2002-Apr-11   Use floor instead of round for percentdone

% 2002-Jun-06   Updated for speed using patch (Thanks to waitbar.m)

% 2002-Jun-19   Choose random patch color when a new figure is created

% 2002-Jun-24   Click on bar or axes to choose new random color

% 2002-Jun-27   Calc time left, reset progress bar when fractiondone == 0

% 2002-Jun-28   Remove extraText var, add position var

% 2002-Jul-18   fractiondone input is optional

% 2002-Jul-19   Allow position to specify screen coordinates

% 2002-Jul-22   Clear vars used in color change callback routine

% 2002-Jul-29   Position input is always specified in pixels

% 2002-Sep-09   Change order of title bar text

% 2003-Jun-13   Change 'min' to 'm' because of built in function 'min'

% 2003-Sep-08   Use callback for changing color instead of string

% 2003-Sep-10   Use persistent vars for speed, modify titlebarstr

% 2003-Sep-25   Correct titlebarstr for 0% case

% 2003-Nov-25   Clear all persistent vars when percentdone = 100

% 2004-Jan-22   Cleaner reset process, don't create figure if percentdone = 100

% 2004-Jan-27   Handle incorrect position input

% 2004-Feb-16   Minimum time interval between updates

% 2004-Apr-01   Cleaner process of enforcing minimum time interval

% 2004-Oct-08   Seperate function for timeleftstr, expand to include days

% 2004-Oct-20   Efficient if-else structure for sec2timestr

%

persistent progfig progpatch starttime lastupdate

% Set defaults for variables not passed in

if nargin < 1

fractiondone = 0;

end

if nargin < 2

position = 0;

end

try

% Access progfig to see if it exists ('try' will fail if it doesn't)

dummy = get(progfig,'UserData');

% If progress bar needs to be reset, close figure and set handle to empty

if fractiondone == 0

delete(progfig) % Close progress bar

progfig = []; % Set to empty so a new progress bar is created

end

catch

progfig = []; % Set to empty so a new progress bar is created

end

% If task completed, close figure and clear vars, then exit

percentdone = floor(100*fractiondone);

if percentdone == 100 % Task completed

delete(progfig) % Close progress bar

clear progfig progpatch starttime lastupdate % Clear persistent vars

return

end

% Create new progress bar if needed

if isempty(progfig)

% Calculate position of progress bar in normalized units

scrsz = [0 0 1 1];

width = scrsz(3)/4;

height = scrsz(4)/50;

if (length(position) == 1)

hpad = scrsz(3)/64; % Padding from left or right edge of screen

vpad = scrsz(4)/24; % Padding from top or bottom edge of screen

left   = scrsz(3)/2 - width/2; % Default

bottom = scrsz(4)/2 - height/2; % Default

switch position

case 0 % Center

% Do nothing (default)

case 1 % Top-right

left   = scrsz(3) - width  - hpad;

bottom = scrsz(4) - height - vpad;

case 2 % Top-left

left   = hpad;

bottom = scrsz(4) - height - vpad;

case 3 % Bottom-left

left   = hpad;

bottom = vpad;

case 4 % Bottom-right

left   = scrsz(3) - width  - hpad;

bottom = vpad;

case 5 % Random

left   = rand * (scrsz(3)-width);

bottom = rand * (scrsz(4)-height);

otherwise

warning('position must be (0-5). Reset to 0.')

end

position = [left bottom];

elseif length(position) == 2

% Error checking on position

if (position(1) < 0) | (scrsz(3)-width < position(1))

position(1) = max(min(position(1),scrsz(3)-width),0);

warning('Horizontal position adjusted to fit on screen.')

end

if (position(2) < 0) | (scrsz(4)-height < position(2))

position(2) = max(min(position(2),scrsz(4)-height),0);

warning('Vertical position adjusted to fit on screen.')

end

else

error('position is not formatted correctly')

end

% Initialize progress bar

progfig = figure(...

'Units',            'normalized',...

'Position',         [position width height],...

'NumberTitle',      'off',...

'Resize',           'off',...

'MenuBar',          'none',...

'BackingStore',     'off' );

progaxes = axes(...

'Position',         [0.02 0.15 0.96 0.70],...

'XLim',             [0 1],...

'YLim',             [0 1],...

'Box',              'on',...

'ytick',            [],...

'xtick',            [] );

progpatch = patch(...

'XData',            [0 0 0 0],...

'YData',            [0 0 1 1],...

'EraseMode',        'none' );

set(progfig,  'ButtonDownFcn',{@changecolor,progpatch});

set(progaxes, 'ButtonDownFcn',{@changecolor,progpatch});

set(progpatch,'ButtonDownFcn',{@changecolor,progpatch});

changecolor(0,0,progpatch)

% Set time of last update to ensure a redraw

lastupdate = clock - 1;

% Task starting time reference

if isempty(starttime) | (fractiondone == 0)

starttime = clock;

end

end

% Enforce a minimum time interval between updates

if etime(clock,lastupdate) < 0.01

return

end

% Update progress patch

set(progpatch,'XData',[0 fractiondone fractiondone 0])

% Update progress figure title bar

if (fractiondone == 0)

titlebarstr = ' 0%';

else

runtime = etime(clock,starttime);

timeleft = runtime/fractiondone - runtime;

timeleftstr = sec2timestr(timeleft);

titlebarstr = sprintf('%2d%%    %s remaining',percentdone,timeleftstr);

end

set(progfig,'Name',titlebarstr)

% Force redraw to show changes

drawnow

% Record time of this update

lastupdate = clock;

% ------------------------------------------------------------------------------

function changecolor(h,e,progpatch)

% Change the color of the progress bar patch

colorlim = 2.8; % Must be <= 3.0 - This keeps the color from being too light

thiscolor = rand(1,3);

while sum(thiscolor) > colorlim

thiscolor = rand(1,3);

end

set(progpatch,'FaceColor',thiscolor);

% ------------------------------------------------------------------------------

function timestr = sec2timestr(sec)

% Convert a time measurement from seconds into a human readable string.

% Convert seconds to other units

d = floor(sec/86400); % Days

sec = sec - d*86400;

h = floor(sec/3600); % Hours

sec = sec - h*3600;

m = floor(sec/60); % Minutes

sec = sec - m*60;

s = floor(sec); % Seconds

% Create time string

if d > 0

if d > 9

timestr = sprintf('%d day',d);

else

timestr = sprintf('%d day, %d hr',d,h);

end

elseif h > 0

if h > 9

timestr = sprintf('%d hr',h);

else

timestr = sprintf('%d hr, %d min',h,m);

end

elseif m > 0

if m > 9

timestr = sprintf('%d min',m);

else

timestr = sprintf('%d min, %d sec',m,s);

end

else

timestr = sprintf('%d sec',s);

end

function progressbar(fractiondone, position)

% Description:

%   progressbar(fractiondone,position) provides an indication of the progress of

% some task using graphics and text. Calling progressbar repeatedly will update

% the figure and automatically estimate the amount of time remaining.

%   This implementation of progressbar is intended to be extremely simple to use

% while providing a high quality user experience.

%

% Features:

%   - Can add progressbar to existing m-files with a single line of code.

%   - The figure closes automatically when the task is complete.

%   - Only one progressbar can exist so old figures don't clutter the desktop.

%   - Remaining time estimate is accurate even if the figure gets closed.

%   - Minimal execution time. Won't slow down code.

%   - Random color and position options. When a programmer gets bored....

%

% Usage:

%   fractiondone specifies what fraction (0.0 - 1.0) of the task is complete.

% Typically, the figure will be updated according to that value. However, if

% fractiondone == 0.0, a new figure is created (an existing figure would be

% closed first). If fractiondone == 1.0, the progressbar figure will close.

%   position determines where the progressbar figure appears on screen. This

% argument only has an effect when a progress bar is first created or is reset

% by calling with fractiondone = 0. The progress bar's position can be specifed

% as follows:

%       [x, y]  - Position of lower left corner in normalized units (0.0 - 1.0)

%           0   - Centered (Default)

%           1   - Upper right

%           2   - Upper left

%           3   - Lower left

%           4   - Lower right

%           5   - Random [x, y] position

%   The color of the progressbar is choosen randomly when it is created or

% reset. Clicking inside the figure will cause a random color change.

%   For best results, call progressbar(0) (or just progressbar) before starting

% a task. This sets the proper starting time to calculate time remaining.

%

% Example Function Calls:

%   progressbar(fractiondone,position)

%   progressbar               % Initialize/reset

%   progressbar(0)            % Initialize/reset

%   progressbar(0,4)          % Initialize/reset and specify position

%   progressbar(0,[0.2 0.7])  % Initialize/reset and specify position

%   progressbar(0.5)          % Update

%   progressbar(1)            % Close

%

% Demo:

%   n = 1000;

%   progressbar % Create figure and set starting time

%   for i = 1:n

%       pause(0.01) % Do something important

%       progressbar(i/n) % Update figure

%   end

%

% Author: Steve Hoelzer

%

% Revisions:

% 2002-Feb-27   Created function

% 2002-Mar-19   Updated title text order

% 2002-Apr-11   Use floor instead of round for percentdone

% 2002-Jun-06   Updated for speed using patch (Thanks to waitbar.m)

% 2002-Jun-19   Choose random patch color when a new figure is created

% 2002-Jun-24   Click on bar or axes to choose new random color

% 2002-Jun-27   Calc time left, reset progress bar when fractiondone == 0

% 2002-Jun-28   Remove extraText var, add position var

% 2002-Jul-18   fractiondone input is optional

% 2002-Jul-19   Allow position to specify screen coordinates

% 2002-Jul-22   Clear vars used in color change callback routine

% 2002-Jul-29   Position input is always specified in pixels

% 2002-Sep-09   Change order of title bar text

% 2003-Jun-13   Change 'min' to 'm' because of built in function 'min'

% 2003-Sep-08   Use callback for changing color instead of string

% 2003-Sep-10   Use persistent vars for speed, modify titlebarstr

% 2003-Sep-25   Correct titlebarstr for 0% case

% 2003-Nov-25   Clear all persistent vars when percentdone = 100

% 2004-Jan-22   Cleaner reset process, don't create figure if percentdone = 100

% 2004-Jan-27   Handle incorrect position input

% 2004-Feb-16   Minimum time interval between updates

% 2004-Apr-01   Cleaner process of enforcing minimum time interval

% 2004-Oct-08   Seperate function for timeleftstr, expand to include days

% 2004-Oct-20   Efficient if-else structure for sec2timestr

%

persistent progfig progpatch starttime lastupdate

% Set defaults for variables not passed in

if nargin < 1

fractiondone = 0;

end

if nargin < 2

position = 0;

end

try

% Access progfig to see if it exists ('try' will fail if it doesn't)

dummy = get(progfig,'UserData');

% If progress bar needs to be reset, close figure and set handle to empty

if fractiondone == 0

delete(progfig) % Close progress bar

progfig = []; % Set to empty so a new progress bar is created

end

catch

progfig = []; % Set to empty so a new progress bar is created

end

% If task completed, close figure and clear vars, then exit

percentdone = floor(100*fractiondone);

if percentdone == 100 % Task completed

delete(progfig) % Close progress bar

clear progfig progpatch starttime lastupdate % Clear persistent vars

return

end

% Create new progress bar if needed

if isempty(progfig)

% Calculate position of progress bar in normalized units

scrsz = [0 0 1 1];

width = scrsz(3)/4;

height = scrsz(4)/50;

if (length(position) == 1)

hpad = scrsz(3)/64; % Padding from left or right edge of screen

vpad = scrsz(4)/24; % Padding from top or bottom edge of screen

left   = scrsz(3)/2 - width/2; % Default

bottom = scrsz(4)/2 - height/2; % Default

switch position

case 0 % Center

% Do nothing (default)

case 1 % Top-right

left   = scrsz(3) - width  - hpad;

bottom = scrsz(4) - height - vpad;

case 2 % Top-left

left   = hpad;

bottom = scrsz(4) - height - vpad;

case 3 % Bottom-left

left   = hpad;

bottom = vpad;

case 4 % Bottom-right

left   = scrsz(3) - width  - hpad;

bottom = vpad;

case 5 % Random

left   = rand * (scrsz(3)-width);

bottom = rand * (scrsz(4)-height);

otherwise

warning('position must be (0-5). Reset to 0.')

end

position = [left bottom];

elseif length(position) == 2

% Error checking on position

if (position(1) < 0) | (scrsz(3)-width < position(1))

position(1) = max(min(position(1),scrsz(3)-width),0);

warning('Horizontal position adjusted to fit on screen.')

end

if (position(2) < 0) | (scrsz(4)-height < position(2))

position(2) = max(min(position(2),scrsz(4)-height),0);

warning('Vertical position adjusted to fit on screen.')

end

else

error('position is not formatted correctly')

end

% Initialize progress bar

progfig = figure(...

'Units',            'normalized',...

'Position',         [position width height],...

'NumberTitle',      'off',...

'Resize',           'off',...

'MenuBar',          'none',...

'BackingStore',     'off' );

progaxes = axes(...

'Position',         [0.02 0.15 0.96 0.70],...

'XLim',             [0 1],...

'YLim',             [0 1],...

'Box',              'on',...

'ytick',            [],...

'xtick',            [] );

progpatch = patch(...

'XData',            [0 0 0 0],...

'YData',            [0 0 1 1],...

'EraseMode',        'none' );

set(progfig,  'ButtonDownFcn',{@changecolor,progpatch});

set(progaxes, 'ButtonDownFcn',{@changecolor,progpatch});

set(progpatch,'ButtonDownFcn',{@changecolor,progpatch});

changecolor(0,0,progpatch)

% Set time of last update to ensure a redraw

lastupdate = clock - 1;

% Task starting time reference

if isempty(starttime) | (fractiondone == 0)

starttime = clock;

end

end

% Enforce a minimum time interval between updates

if etime(clock,lastupdate) < 0.01

return

end

% Update progress patch

set(progpatch,'XData',[0 fractiondone fractiondone 0])

% Update progress figure title bar

if (fractiondone == 0)

titlebarstr = ' 0%';

else

runtime = etime(clock,starttime);

timeleft = runtime/fractiondone - runtime;

timeleftstr = sec2timestr(timeleft);

titlebarstr = sprintf('%2d%%    %s remaining',percentdone,timeleftstr);

end

set(progfig,'Name',titlebarstr)

% Force redraw to show changes

drawnow

% Record time of this update

lastupdate = clock;

% ------------------------------------------------------------------------------

function changecolor(h,e,progpatch)

% Change the color of the progress bar patch

colorlim = 2.8; % Must be <= 3.0 - This keeps the color from being too light

thiscolor = rand(1,3);

while sum(thiscolor) > colorlim

thiscolor = rand(1,3);

end

set(progpatch,'FaceColor',thiscolor);

% ------------------------------------------------------------------------------

function timestr = sec2timestr(sec)

% Convert a time measurement from seconds into a human readable string.

% Convert seconds to other units

d = floor(sec/86400); % Days

sec = sec - d*86400;

h = floor(sec/3600); % Hours

sec = sec - h*3600;

m = floor(sec/60); % Minutes

sec = sec - m*60;

s = floor(sec); % Seconds

% Create time string

if d > 0

if d > 9

timestr = sprintf('%d day',d);

else

timestr = sprintf('%d day, %d hr',d,h);

end

elseif h > 0

if h > 9

timestr = sprintf('%d hr',h);

else

timestr = sprintf('%d hr, %d min',h,m);

end

elseif m > 0

if m > 9

timestr = sprintf('%d min',m);

else

timestr = sprintf('%d min, %d sec',m,s);

end

else

timestr = sprintf('%d sec',s);

end

function quiverplot

% Create a plot showing motion vectors for every P frame in a movie.

load lastmov

[M,N] = size(mpeg{1});

for f = 1:length(mpeg)

if mpeg{f}(1,1).type == 'I'

continue

end

for i = 1:M

for j = 1:N

mvx(i,j) = mpeg{f}(i,j).mvy;

mvy(i,j) = mpeg{f}(i,j).mvx;

end

end

figuresc(0.8)

quiver(flipud(mvx),flipud(mvy))

set(gca,'XLim',[-1, N+2],'YLim',[-1, M+2])

title(sprintf('Motion vectors for frame %i',f))

end

%%%%%%=====================================================================

function quiverplot

% Create a plot showing motion vectors for every P frame in a movie.

load lastmov

[M,N] = size(mpeg{1});

for f = 1:length(mpeg)

if mpeg{f}(1,1).type == 'I'

continue

end

for i = 1:M

for j = 1:N

mvx(i,j) = mpeg{f}(i,j).mvy;

mvy(i,j) = mpeg{f}(i,j).mvx;

end

end

figuresc(0.8)

quiver(flipud(mvx),flipud(mvy))

set(gca,'XLim',[-1, N+2],'YLim',[-1, M+2])

title(sprintf('Motion vectors for frame %i',f))

end

function timestr = sec2timestr(sec)

% Convert a time measurement from seconds into a human readable string.

%

% Examples:

%   sec2timestr(1)       => '1 sec'

%   sec2timestr(11)      => '11 sec'

%   sec2timestr(111)     => '1 min, 51 sec'

%   sec2timestr(1111)    => '18 min'

%   sec2timestr(11111)   => '3 hr, 5 min'

%   sec2timestr(111111)  => '1 day, 6 hr'

%   sec2timestr(1111111) => '12 day'

%

% Change log:

% 2004-Oct-08 - Steve Hoelzer - Conception

% 2004-Oct-20 - Steve Hoelzer - Efficient if-else structure

%

% Convert seconds to other units

d = floor(sec/86400); % Days

sec = sec - d*86400;

h = floor(sec/3600); % Hours

sec = sec - h*3600;

m = floor(sec/60); % Minutes

sec = sec - m*60;

s = floor(sec); % Seconds

% Create time string

if d > 0

if d > 9

timestr = sprintf('%d day',d);

else

timestr = sprintf('%d day, %d hr',d,h);

end

elseif h > 0

if h > 9

timestr = sprintf('%d hr',h);

else

timestr = sprintf('%d hr, %d min',h,m);

end

elseif m > 0

if m > 9

timestr = sprintf('%d min',m);

else

timestr = sprintf('%d min, %d sec',m,s);

end

else

timestr = sprintf('%d sec',s);

end

%%%==================================================================

function timestr = sec2timestr(sec)

% Convert a time measurement from seconds into a human readable string.

%

% Examples:

%   sec2timestr(1)       => '1 sec'

%   sec2timestr(11)      => '11 sec'

%   sec2timestr(111)     => '1 min, 51 sec'

%   sec2timestr(1111)    => '18 min'

%   sec2timestr(11111)   => '3 hr, 5 min'

%   sec2timestr(111111)  => '1 day, 6 hr'

%   sec2timestr(1111111) => '12 day'

%

% Change log:

% 2004-Oct-08 - Steve Hoelzer - Conception

% 2004-Oct-20 - Steve Hoelzer - Efficient if-else structure

%

% Convert seconds to other units

d = floor(sec/86400); % Days

sec = sec - d*86400;

h = floor(sec/3600); % Hours

sec = sec - h*3600;

m = floor(sec/60); % Minutes

sec = sec - m*60;

s = floor(sec); % Seconds

% Create time string

if d > 0

if d > 9

timestr = sprintf('%d day',d);

else

timestr = sprintf('%d day, %d hr',d,h);

end

elseif h > 0

if h > 9

timestr = sprintf('%d hr',h);

else

timestr = sprintf('%d hr, %d min',h,m);

end

elseif m > 0

if m > 9

timestr = sprintf('%d min',m);

else

timestr = sprintf('%d min, %d sec',m,s);

end

else

timestr = sprintf('%d sec',s);

end